BD 48

ELECTRONIC BLOOD DRAWING CHAIR, 4 MOTORS

- 4 DC motors

- Electronic adjustments of backrest, legrest, height, and Trendelenburg functions by hand control unit

- The chair is upholstered by high density foam that distributes the body pressure over a wide area and besides comfortable seating, allows for good blood circulation

- Foam’s capacity let the patient redistribute weight and surface pressure, potentially reducing decubitus ulcers

- ABS covering designed to keep itself in the upholstery and it blocks the dust which goes into the upholstery

- During the back rest movement, arm rests moves with the back rest simultaneously and the arm rests are easy removable

- Easy adjustable and removable upholstered arm rests

- 360° swivel, diagonal lockable castors of 125 mm diameter

- Rechargeable Back-up battery (optional)

TECHNICAL DETAILS

Backrest Angle	: 0°-75°

Trendelenburg Angle	: 0°-13°

Reverse Trendelenburg Angle	: 0°-(-3)°

Legrest Angle	: 0°-85°

Safe Working Load Capacity	: 180 kg

Weight	: 105 kg

External Dimensions (with Armrest)	: 174 x 102 cm

Bed Surface	: 174 x 54.5 cm

Height Range	: 64-84 cm

Castor Diameter	: 125 mm

Power In	: Max. 3,15A 100V-240V ~ 50-60 Hz

Electrical Shock Protection	: Class I, Type B

Liquid Ingress Protection	: IPX 4