CL 45

ELECTRONIC ICU BED, COLUMN MOTORS

- Columnar Motors

- Electrical operated backrest, height and legrest adjustment

- Electrical operated Trendelenburg, Reverse Trendelenburg, fowler and vascular position

- Footrest adjustment by ratchet

- Remote control for adjusting positions

- Auto-CPR

- Auto Regression system on backrest

- Auto contour

- Hygienic design (PP), lockable and tuck away side rails

- Easy removable ABS mattress platform

- X-ray translucent HPL backrest

- Removable head and footboards

- Electrostatic epoxy painted metal frame

- Height adjustable stainless steel IV pole

- Plastic crash bumpers

- Central Lockable Castors, 150 mm

- Angle indicator for backrest

- Angle indicator for Trendelenburg and reverse Trendelenburg positions

- Cardiac chair position

- Shock position

- Semi-Fowler position

- Bed exit position

- Examination position

- CPR activation with single button

- Foot end nurse control panel

- Control panels embedded on side rails; 2 for patient; 2 for nursing staff

- Activation key at remote control to prevent any undesired positions

- Dual sided manual CPR levers at backrest

- Rechargeable Battery Backup

- X-ray cassette holder

- Urine bag holder

- Integrated weight scale system (Optional)

- Bed size extension (optional)

- IPX6 standard electronic system

TECHNICAL DETAILS

Overall Length	: 220 cm

Overall Width	: 100 cm

Mattress Surface	: 85 x 200 cm

Height	: 48-86 cm

Trendelenburg	: 0°-15°

Reverse Trendelenburg	: 0°-15°

Backrest Angle	: 0°-70°

Legrest Angle	: 0°-40°

Castor Diameter	: 15 cm

Under bed Clearance	: 15 cm

Safe Working Load	: 250 kg

Power In	: 1.5A max. At 230V AC 50-60 Hz

Electrical Shock Protection	: Class I, Type B

Liquid Ingress Protection	: IPX6

Batteries	: 2x 12V Series Connected, sealed, rechargeable, Lead / Acid gel, 1.2 amp hrs.