CL 55

ELECTRONIC ICU BED, 5 MOTORS

- Electrically operated backrest, height, and legrest adjustment

- Electrically operated Trendelenburg, Reverse Trendelenburg, Fowler, and vascular position

- Lateral tilt functionality for enhanced patient positioning

- Foot end nurse control unit

- Control panels embedded on side rails: 2 for patients, 2 for nursing staff

- Footrest adjustment by ratchet

- Auto-CPR

- Auto Regression system on backrest

- Auto contour

- Hygienic design (PP), lockable, and tuck-away side rails

- Easy removable ABS mattress platform

- X-ray cassette holder

- Removable headboard and footboard

- Electrostatic epoxy-painted metal frame

- Height-adjustable stainless steel IV pole

- Plastic protective crash bumpers on four corners

- Central Lockable Castors, 150 mm

- Angle indicator for backrest

- Angle indicator for Trendelenburg and Reverse Trendelenburg positions

- Cardiac chair position with single button

- Shock position with single button

- Semi-Fowler position with single button

- Bed exit position with single button

- Examination position with single button

- Electronic CPR with one-touch button

- Foot switch control for additional convenience

- Activation key at remote control to prevent undesired positions

- Dual-sided manual CPR levers at backrest

- Urine bag holder

- Rechargeable battery backup

- Bed size extension (optional)

- X-ray cassette holder at backrest

- IPX6 standard electronic system

TECHNICAL DETAILS

Overall Length	: 220 cm

Overall Width	: 100 cm

Mattress Surface	: 85 x 200 cm

Height	: 48-86 cm

Trendelenburg	: 0°-17°

Reverse Trendelenburg	: 0°-17°

Lateral Tilt (Right)	: 0°-13°

Lateral Tilt (Left)	: 0°-13°

Backrest Angle	: 0°-70°

Legrest Angle	: 0°-40°

Castor Diameter	: 15 cm

Under bed Clearance	: 15 cm

Safe Working Load	: 250 kg

Power In	: 1.5A max. At 230V AC 50-60 Hz

Electrical Shock Protection	: Class I, Type B

Liquid Ingress Protection	: IPX6

Batteries	: 2x 12V Series Connected, sealed, rechargeable, Lead / Acid gel, 1.2 amp hrs.