IN 42

ELECTRONIC ICU BED, 4 MOTORS

Click for detailed video of the product

- Electrical operated backrest, height and legrest adjustment

- Electrical operated Trendelenburg, Reverse Trendelenburg, fowler and vascular position

- Patient handset

- Foot end nurse control unit

- Footrest adjustment by ratchet

- Auto-CPR

- Auto Regression system on backrest

- Auto contour

- Hygienic design (PP), lockable and tuck away side rails

- Easy removable ABS mattress platform

- Removable head and footboards

- Electrostatic epoxy painted metal frame

- Height adjustable IV pole

- Plastic protective crash bumpers on four corners

- Central Lockable Castors, 150 mm, Single

- Angle indicator for backrest

- Angle indicator for Trendelenburg and Reverse Trendelenburg positions

- Cardiac chair position with single button

- Shock position with single button

- Semi-Fowler position with single button

- Bed exit position with single button

- Examination position with single button

- Electronic CPR with single button

- Activation key at remote control to prevent any undesired positions

- Urine bag holder

- Urine basket (optional)

- Dual side manual CPR levers at backrest (optional)

- Rechargeable Battery Backup (optional)

- Bed size extension (optional)

- Twin Castors (optional)

- X-ray cassette holder at backrest (optional)

- IPX6 standard electronic system

|  | TECHNICAL DETAILS |  |
| --- | --- | --- |
|  | Overall Length: 216 cm | Castor Diameter: 15 cm |
|  | Overall Width: 102 cm | Under Bed Clearance: 15 cm |
|  | Height: 42-84 cm | Product Weight: 115 kg |
|  | Trendelenburg : 0°-16° | Safe Working Load: 250 kg |
|  | Reverse Trendelenburg: 0°-16° | Power In: 1.5A max. At 230V AC 50-60 Hz |
|  | Backrest Angle: 0°-70° | Electrical Shock Protection: Class I, Type B |
|  | Legrest Angle: 0°-35° | Liquid Ingress Protection: IPX6 |