Brand: CA-MI

Model: ASKIR 36BR

Name: ASKIR 36BR is a portable suction unit

Description

ASKIR 36BR is a portable suction unit for the aspiration of body liquids, oral, nasal and tracheal aspiration in adults or children.
For professional use and with a powerful aspiration of max 36 l/min, ASKIR 36BR is lightweight for easy transport.
Three different options for operation: AC/DC adapter, internal rechargeable battery and 12V make it a versatile suction unit approved for NON-STOP operation without overheating.
Provided with visual and acoustic alarm indicating low battery level.
Available with Lead-type battery or Li-ion battery.
The Li-ion battery model is equipped with a pressure sensor placed in the PCB that reduces by 50% the speed of the motor when no vacuum is detected and therefore no aspiration is taking place, and PROXIMITY function to switch ON/OFF without touch.

Main Applications

- Ambulance Car

- Emergency Department

- Home Care

- Patient Transfer

- Room / Ward

- Tracheostomy

- Veterinary

Accessories Included

- Liquid collection jar with overflow valve system (multiple options available)

- 8×14 mm silicone tubes (150 cm, autoclavable) with conical connector (10–11–12 mm)

- Antibacterial & hydrophobic filter (single-patient use)

- Power cord + 12V car adapter + AC/DC universal adapter

Optional:

- Stand on 5 castors Ø 50 mm with brakes (antistatic upon request)

- Carrying bag (1000 ml / 2000 ml options)

Technical Features

ASKIR 36BR (Lead Battery)

- Motor: Oil-less, maintenance-free piston pump

- Power Feeding:

  - 14V (4A 100–240V~)

  - 50–60Hz AC/DC adapter

  - 12V car adapter

- Battery: Lead rechargeable – autonomy up to 60 min (charging time ~4 hours)

- Classification: High Vacuum / High Flow

- IP Code: IP21

- Max Vacuum (adjustable): -0.80 bar / -80 kPa / -600 mmHg

- Max Air Flow: 36 L/min

- Noise Level: 65.5 dB(A)

- Duty Cycle: Non-stop operation (by mains only)

- Weight: 4.39 kg

- Dimensions: 35 × 21 × 18 cm

- Warranty: 2 years

- Manufacturing: Italy

NEW ASKIR 36 Li-ION (Proximity)

- Motor: Oil-less, maintenance-free piston pump

- Power Feeding:

  - 14V (4A 100–240V~)

  - 50–60Hz AC/DC adapter

  - 12V car adapter

- Battery: 14.8V – 5.2A Li-ion rechargeable (autonomy up to 70 min, charging time ~6 hours)

- ON/OFF: Soft-touch + Proximity sensor

- Classification: High Vacuum / High Flow

- IP Code: IP21

- Max Vacuum (adjustable): -0.80 bar / -80 kPa / -600 mmHg

- Max Air Flow: 36 L/min

- Noise Level: 60 dB(A)

- Duty Cycle: Non-stop operation (by mains only)

- Weight: 3.00 kg

- Dimensions: 35 × 21 × 18 cm

- Warranty: 2 years

- Manufacturing: Italy