UT 26

EMERGENCY TROLLEY

|  | Construction: | Electrostatic powder painted metal frame and ABS |
| --- | --- | --- |
|  | Body Material: | Side walls and drawers electrostatic powered painted steel sheet Working top and removable drawers trays ABS material |
|  | Knocking Mechanism: | Central locking system with key Optional: Breakaway seal system |
|  | Top Surface: | ABS work top with raised edges on three sides to prevent falling |
|  | Drawers: | 4 medium with 120 mm height + 1 big 220 mm height, total 5 drawers. Optional: 2 medium with 120 mm height + 2 big 220 mm height, total 4 drawers. Built in drawer handles with different colours |
|  | Drawer Dimensions: | 4 pcs medium drawers : 355 x 555 x 100 mm 1 pcs big drawer: 355 x 555 x 200 mm Inner dimensions |
|  | Inside Drawers: | Made by ABS material, easy detachable trays Each drawer totally 2 separators for 4 compartments |
|  | Pull-put Working Tray: | High quality and rigid stainless steel working tray for medical staff. 304 grade steel tray with railing system 295 x 350 mm dimension |
|  | Castors: | 2 pcs lockable , 2 pcs without lock ,125 mm 360° swivel castors, made of non-marking material |
|  | Defibrillator Holder: | Stainless steel, rotatable, height adjustable. Assembled with security Velcro belt. 300 x 350 mm dimensions (L x W) |
|  | Oxygen Tube Holder: | Made by PP epoxy painted steel. Supplied security belt holder. 5 litres capacity |
|  | Waste Bin: | Detachable bin, made by PE material |
|  | IV Hook: | 4 hooks, IV pole, height adjustable |
|  | Resuscitation Board: | Located on the backside with holding from there ends High quality plexiglass material 395 x 600 mm dimension. 8 mm thickness |
|  | Label Holder: | Label holder on each drawer for easy identification |
|  | Electric Crunch: | (Optional) |

|  | TECHNICAL DETAILS |  |
| --- | --- | --- |
|  | Body Dimension | : 700 x 550 mm |
|  | External Dimension (with Accessories) | : 980 x 700 mm |
|  | Height | : 1250 mm |
|  | Castor Diameter | : 125 mm |

OPTIONAL ACCESSORIES

Sharp Box

Glove Box Holder

Plastic Bumpers

Break-Away Seal System