CL 45

ELECTRONIC ICU BED, COLUMN MOTORS

- Columnar Motors

- Electrical operated backrest, height and legrest adjustment

- Electrical operated Trendelenburg, Reverse Trendelenburg, fowler and vascular position

- Footrest adjustment by ratchet

- Remote control for adjusting positions

- Auto-CPR

- Auto Regression system on backrest

- Auto contour

- Hygienic design (PP), lockable and tuck away side rails

- Easy removable ABS mattress platform

- X-ray translucent HPL backrest

- Removable head and footboards

- Electrostatic epoxy painted metal frame

- Height adjustable stainless steel IV pole

- Plastic crash bumpers

- Central Lockable Castors, 150 mm

- Angle indicator for backrest

- Angle indicator for Trendelenburg and reverse Trendelenburg positions

- Cardiac chair position

- Shock position

- Semi-Fowler position

- Bed exit position

- Examination position

- CPR activation with single button

- Foot end nurse control panel

- Control panels embedded on side rails; 2 for patient; 2 for nursing staff

- Activation key at remote control to prevent any undesired positions

- Dual sided manual CPR levers at backrest

- Rechargeable Battery Backup

- X-ray cassette holder

- Urine bag holder

- Integrated weight scale system (Optional)

- Bed size extension (optional)

- IPX6 standard electronic system

|  | TECHNICAL DETAILS |  |
| --- | --- | --- |
|  | Overall Length | : 220 cm |
|  | Overall Width | : 100 cm |
|  | Mattress Surface | : 85 x 200 cm |
|  | Height | : 48-86 cm |
|  | Trendelenburg | : 0°-15° |
|  | Reverse Trendelenburg | : 0°-15° |
|  | Backrest Angle | : 0°-70° |
|  | Legrest Angle | : 0°-40° |
|  | Castor Diameter | : 15 cm |
|  | Under bed Clearance | : 15 cm |
|  | Safe Working Load | : 250 kg |

|  | ELECTRICAL DATA |  |
| --- | --- | --- |
|  | Power In | : 1.5A max. At 230V AC 50-60 Hz |
|  | Electrical Shock Protection | : Class I, Type B |
|  | Liquid Ingress Protection | : IPX6 |
|  | Batteries | : 2x 12V Series Connected, sealed, rechargeable, Lead / Acid gel, 1.2 amp hrs. |

OPTIONAL ACCESSORIES

Central Brake Castors, 150 mm

Foot-end Nurse Control Panel

Rechargeable Battery Backup

X-Ray Cassette Holder at Backrest

Dual Sided Manual CPR at Backrest

Control Panels Embedded on Side Rails

Bed Extension

Oxygen Cylinder Holder

Urine Bag Basket

Stationary Headboard

Defibrillator Shelf

Nurse Panel Shelf

Orthopedic Traction System

Linen Holder

Lifting Pole

Weight Scale System

Touch Screen foot-end bed

Double Wheels

Foot Switch

5th Castor

COLOR CHART

P01 – White

P02 – Cappuccino

P03 – Light Blue

P04 – Green

P05 – Light Gray

P06 – Navy Blue

P07 – Dark Gray

P08 – Yellow

P09 – Aquamarine

P10 – Claret Red