MOMENTUM

FUNCTIONAL TRAUMA STRETCHER

- Hydraulic height adjustment supported with columns

- Height adjustment pedals on both sides

- 3 sections mattress surface

- Trendelenburg and Reverse Trendelenburg positions

- Backrest and legrest adjustment assisted by gas spring

- Foam mattress with Removable, anti-static and fire-retardant, waterproof and compatible with the size of the stretcher, 8 cm mattress thickness

- Full-length dual deck HPL radio-translucent mattress platform allows X-Ray filming all over the stretcher

- Oxygen tank holder (12 liters)

- Full-length foldable and lockable side rails. Easy storable under the mattress providing minimum transfer gap

- Foldable PU Pushing handles provide easy maneuverability

- Electrostatic powder (epoxy) coated main frame

- ABS sub-frame cover ensure maximum sterilization

- Dual functional pedal for central and directional lock at head and foot sides

- Height adjustable and foldable stainless steel IV Pole

- Urine bag hanger hooks

- Anti-static and anti-tracing, 360° Swivel castors

- 200mm Castors with central braking mechanism and steer control

- 125 mm dia. 5th Wheel for easy maneuverability (optional)

- Paper Roll hanger (optional)

- Patient safety belt (optional)

- Monitor Shelf (optional)

- Viscoelastic Mattress (optional)

Momentum Stretcher and Emergeum Emergency Stretcher

|  | TECHNICAL DETAILS |  |
| --- | --- | --- |
|  | Length | : 205 cm |
|  | Width | : 85 cm |
|  | Backrest Angle | : 0°-70° |
|  | Legrest Angle | : 0°-40° |
|  | TR/RTR Angle | : 0°-16° |
|  | Height (Without Mattress) | : 65-95 cm |
|  | Castor Diameter | : 200 mm |
|  | 5th Castor Diameter (optional) | : 125 mm |
|  | Mattress Surface | : 62,5 x 190 cm |
|  | Mattress Thickness | : 8 cm |
|  | Safe Working Load | : 250 kg |