UT 27

DRESSING AND PROCEDURE TROLLEY

|  | Construction: | Electrostatic powder painted metal frame and ABS |
| --- | --- | --- |
|  | Body Material: | Side walls and drawers electrostatic powered painted steel sheet Working top and removable drawers trays ABS material |
|  | Knocking Mechanism: | Central locking system with key |
|  | Top Surface: | ABS work top with raised edges on three sides to prevent falling |
|  | Drawers: | 1 medium with 120 mm height + 1 big 220 mm height Optional: 3 mediums with 120 mm height Built in drawer handles with different colours |
|  | Drawer Dimensions: | 1 pcs medium drawer: 355 x 555 x 100 mm 1 pcs big drawer: 355 x 555 x 200 m Inner dimensions |
|  | Inside Drawers: | Made by ABS material, easy detachable trays Each drawer totally 2 separators for 4 compartments |
|  | Pull-put Working Tray: | High quality and rigid stainless steel working tray for medical staff 304 grade steel tray with railing system 295 x 350 mm dimension |
|  | Medicine Boxes: | 10 pcs of, tilt-out, and transparent medicine boxes |
|  | Castors: | 2 pcs lockable, 2 pcs without lock, 125 mm 360° swivel castors, made of non-marking material |
|  | Waste Bin: | Detachable bin, made by PE material |
|  | IV Hook: | Stainless steel, 4 hooks, IV pole, height adjustable |
|  | Label Holder: | Label holder on each drawer for easy identification |

|  | TECHNICAL DETAILS |  |
| --- | --- | --- |
|  | Body Dimensions | : 700×550 mm |
|  | Height | : 1150 mm |
|  | Castor Diameter | : 125 mm |

OPTIONAL ACCESSORIES

Sharp Box

Glove Box Holder

Plastic Bumpers

10 pcs. Medicine Box