CL 43

ELECTRONIC ICU BED, 4 MOTORS

- Electrical operated backrest, height and legrest adjustment

- Electrical operated Trendelenburg, Reverse Trendelenburg, fowler and vascular position

- Foot end nurse control unit

- Control panels embedded on side rails; 2 for patient; 2 for nursing staff

- Footrest adjustment by ratchet

- Auto-CPR

- Auto Regression system on backrest

- Auto contour

- Hygienic design (PP), lockable and tuck away side rails

- Easy removable ABS mattress platform

- X-ray cassette holder

- Removable head and footboards

- Electrostatic epoxy painted metal frame

- Height adjustable stainless steel IV pole

- Plastic protective crash bumpers on four corners

- Central Lockable Castors, 150 mm

- Angle indicator for backrest

- Angle indicator for Trendelenburg and Reverse Trendelenburg positions

- Cardiac chair position with single button

- Shock position with single button

- Semi-Fowler position with single button

- Bed exit position with single button

- Examination position with single button

- Electronic CPR with one touch button

- Activation key at remote control to prevent any undesired positions

- Dual sided manual CPR levers at backrest

- Urine bag holder

- Rechargeable Battery Backup

- Bed size extension

- X-ray cassette holder at backrest

- IPX6 standard electronic system

TECHNICAL DETAILS

Overall Length	: 220 cm

Overall Width	: 100 cm

Mattress Surface	: 85 x 200 cm

Height	: 37-80 cm

Trendelenburg	: 0°-17°

Reverse Trendelenburg	: 0°-17°

Backrest Angle	: 0°-70°

Legrest Angle	: 0°-40°

Castor Diameter	: 15 cm

Under bed Clearance	: 15 cm

Safe Working Load	: 250 kg

Power In	: 1.5A max. At 230V AC 50-60 Hz

Electrical Shock Protection	: Class I, Type B

Liquid Ingress Protection	: IPX6

Batteries	: 2x 12V Series Connected, sealed, rechargeable, Lead / Acid gel, 1.2 amp hrs.