IN 45

ELECTRONIC ICU BED, COLUMN MOTORS

Click for detailed video of the product

- Linak brand Columnar Motors

- Electrical operated backrest, height and legrest adjustment

- Electrical operated Trendelenburg, Reverse Trendelenburg, fowler and vascular position

- Footrest adjustment by ratchet

- Remote control for adjusting positions

- Auto-CPR

- Auto Regression system on backrest

- Auto contour

- Hygienic design (PP), lockable and tuck away side rails

- Easy removable ABS mattress platform

- X-ray translucent HPL backrest

- Removable head and footboards

- Electrostatic epoxy painted metal frame

- Height adjustable stainless steel IV pole

- Plastic crash bumpers

- Central Lockable Castors, 150 mm

- Angle indicator for backrest

- Angle indicator for Trendelenburg and reverse Trendelenburg positions

- Cardiac chair position

- Shock position

- Semi-Fowler position

- Bed exit position

- Examination position

- CPR activation with single button

- Foot end nurse control panel

- Control panels embedded on side rails; 2 for patient; 2 for nursing staff

- Activation key at remote control to prevent any undesired positions

- Dual sided manual CPR levers at backrest

- Rechargeable Battery Backup

- X-ray cassette holder

- Urine bag holder

- Integrated Linak brand weight scale system (Optional)

- Bed size extension (optional)

- IPX6 standard electronic system

Other Electronic icu bed

|  | TECHNICAL DETAILS |  |
| --- | --- | --- |
|  | Overall Length: 216 cm | Under Bed Clearance: 15 cm |
|  | Overall Width: 102 cm | Product Weight: 130 kg |
|  | Height: 44-84 cm | Safe Working Load: 250 kg |
|  | Trendelenburg : 0°-16° | Power In: 1.5A max. At 230V AC 50-60 Hz |
|  | Reverse Trendelenburg: 0°-16° | Electrical Shock Protection: Class I, Type B |
|  | Backrest Angle: 0°-70° | Liquid Ingress Protection: IPX6 |
|  | Legrest Angle: 0°-35° | Batteries: 2x 12V Series Connected, sealed, rechargeable, Lead / Acid gel,1.2 amp hrs. |
|  | Castor Diameter: 15 cm |  |