EC 17

ENT AND EYE EXAMINATION CHAIR, 4 MOTORS

- 4 DC motors.

- Suitable for ENT and eye examination purposes

- Electronic adjustments of backrest, leg rest, height, and Trendelenburg functions by hand control unit

- The head rest is removable and adjustable in multiple positions

- Anti-microbial, water proof, flame retardant, air permeable and easy cleanable cushion and not effected to skin

- The chair the upholstery is made of high density foam of 30 kg/m3 with Artificial leather cover

- Foam’s capacity let the patient redistribute weight and surface pressure, potentially reducing decubitus ulcers

- ABS covering designed to keep itself in the upholstery and it blocks the dust which goes into the upholstery

- During the back rest movement, arm rests moves with the back rest simultaneously and the arm rests are easy removable

- Easy adjustable and removable upholstered arm rests

- The chair base is mounted on adjustable feet

TECHNICAL DETAILS

Backrest Angle	: 0°-78°

Trendelenburg Angle	: 0°-12°

Legrest Angle	: 0°-88°

Headrest Angle	: 0°-90°

Safe Working Load Capacity	: 180 kg

Weight	: 110 kg

External Dimensions (with Armrest)	: 1959 x 640 mm

Height Range	: 640-840 mm

Power In	: Max. 3,15A 100V-240V ~ 50-60 Hz

Electrical Shock Protection	: Class I, Type B

Liquid Ingress Protection	: IPX 4